PAINTFACTOR DOCTRINE

Painted Doors - Residential

| Version | 1.0.0 |
| --- | --- |
| Status | Draft - Pending Approval |
| Date | 2026-01-27 |
| Source | NotebookLM Research (47 sources) |
| Target Specs | door_interior_flush, door_interior_panel, door_exterior_entry, door_specialty_french, door_specialty_bifold |

Critical: PaintScope Integration: Door specs consume geometry from PaintScope. Specs must NOT compute quantities - only reference surface types and apply labor/material factors.

# 1. Scope & Definitions

## 1.1 Door Types Covered

| Door Type | Complexity | Unit | Notes |
| --- | --- | --- | --- |
| Flush (hollow core) | 1.0 | EA (2 sides) | Baseline reference |
| Flush (solid core) | 1.0 | EA (2 sides) | Same complexity, heavier |
| Panel (2-panel) | 1.15 | EA (2 sides) | Detail work on panels |
| Panel (4-panel) | 1.25 | EA (2 sides) |  |
| Panel (6-panel) | 1.35 | EA (2 sides) | Colonial style, most common |
| Raised panel | 1.4 | EA (2 sides) | Shadow lines, more detail |
| French (10-lite) | 1.8 | EA (2 sides) | Glass masking intensive |
| French (15-lite) | 2.2 | EA (2 sides) |  |
| Bifold (flush) | 0.7 | EA per panel | Smaller panels, track work |
| Bifold (louvered) | 1.5 | EA per panel | Louver complexity +10-15% |
| Louver/closet | 1.8 | EA (2 sides) | Spray preferred |
| Dutch door | 1.5 | EA (2 sides) | Two sections |
| Entry (decorative) | 1.5-2.5 | EA | Varies by detail level |

## 1.2 Measurement Conventions  The standard convention needs to be per side.  This eliminates the need for special conventions.

| Convention | Definition | Use Case |
| --- | --- | --- |
| EA (per door) | Complete door, both sides, all edges | Standard bidding unit |
| Per side | One face only | One side accessible/required |
| SF | Square footage (~35 SF per standard door) | Large/unusual doors |

## 1.3 Interior vs Exterior Classification

| Attribute | Interior | Exterior |
| --- | --- | --- |
| Coating system | Standard latex/waterborne alkyd | UV-resistant, flexible coatings |
| Prep intensity | Lower | Higher (weathering assessment) |
| Edge treatment | Standard | Critical (moisture barrier) |
| Quality floor | QT2 acceptable | QT3 minimum QT2 acceptable(without warranty and disclaimer) |
| DFT requirement | 2.0-3.0 mils | 3.0-4.0 mils |
| Color restrictions | None | Dark colors increase thermal stress |

# 2. Substrate Classification

## 2.1 Substrate Types

| Substrate | Applications | Key Considerations |
| --- | --- | --- |
| Solid wood (pine, poplar) | Premium interior, entry | Grain filling, knot sealing required |
| MDF (molded panel) | Interior paint-grade | Edge sealing critical; no grain/knots |
| Hollow core | Budget interior | Light sanding only; fragile |
| Fiberglass (smooth) | Exterior entry | Adhesion primer required; warp-resistant |
| Fiberglass (textured) | Exterior, wood-look | Can be stained or painted |
| Steel (20-24 gauge) | Exterior, fire-rated | Rust prevention critical |
| Composite | Newer exterior options | Per manufacturer specs |

## 2.2 Factory Primed Assessment

| Condition | Required Action |
| --- | --- |
| Intact, clean | Scuff sand 180-grit, proceed (field primer recommended) |
| Damaged/scratched | Spot prime damaged areas, sand blend |
| Weathered/chalking | Full sand, full prime |
| Unknown origin | Test adhesion, bonding primer if doubtful |

Critical: Factory primer is transit protection, not finish-ready primer. Always assess and typically apply field primer for quality work.

# 3. Surface Preparation Requirements

## 3.1 New Construction - By Substrate

| Substrate | Cleaning | Sanding | Sealing | Notes |
| --- | --- | --- | --- | --- |
| Solid wood | Dust removal | 150-grit | Shellac for knots | Grain raise risk |
| MDF | Dust removal | 120-150 edges | Seal all edges | Edges porous |
| Hollow core | Dust removal | Light 180 | Standard | No heavy pressure |
| Fiberglass | Degreaser | Scuff pad | Bonding primer | Remove mold-release |
| Steel | Degreaser | 120-150 | DTM primer | Apply immediately |

## 3.2 Repaint Preparation

| Existing Condition | Required Prep |
| --- | --- |
| Sound paint, good adhesion | Clean with TSP/degreaser, scuff sand 150-220 grit |
| Peeling/flaking | Scrape loose, feather edges, spot prime |
| Multiple coats (buildup) | Assess for stripping or heavy sand |
| Unknown coating | Adhesion test, bonding primer if needed |
| Gloss/semi-gloss | Deglosser or thorough sand for tooth |

## 3.3 Contaminant Removal

Doors are high-touch surfaces accumulating oils, cleaning chemicals, and dust. Failure to remove contaminants causes fisheying or premature delamination.

- Wash with TSP solution or specialized degreaser

- Rinse with clean water

- Allow complete drying (TSP residue interferes with acrylic curing)

# 4. Primer Systems by Substrate

| Substrate | Primer Type | Purpose | Notes |
| --- | --- | --- | --- |
| Solid wood | Stain-blocking primer | Seal, block tannins | Shellac for knots |
| MDF (edges) | Shellac or oil-based | Seal porous edges | Water causes swelling |
| MDF (faces) | High-build latex | Build film, level | After edge seal |
| Hollow core | Standard latex | Seal, provide tooth | Light application |
| Fiberglass | Bonding primer | Adhesion to slick surface | Remove mold-release first |
| Steel | DTM primer | Rust inhibition | Apply immediately |

## 4.1 Primer Functions

- Seal the substrate - Prevent uneven absorption

- Provide tooth - Mechanical adhesion for topcoat

- Block stains/tannins - Prevent bleed-through

# 5. Finish Coat Systems

## 5.1 Interior Door Coatings

| Coating Type | Benefits | Limitations | Use |
| --- | --- | --- | --- |
| Standard Latex | Fast dry, low odor | Lower hardness, blocking | QT2 budget |
| Waterborne Alkyd | Excellent leveling, hard | Longer cure time | QT3-QT4 standard |
| Urethane-Modified | Superior durability | Higher cost | High-traffic, QT4+ |
| Water-Based Urethane | Hardness of oil | More expensive | Kitchen/bath |

## 5.2 Exterior Door Requirements

- UV resistance: Titanium dioxide pigmentation; UV absorbers

- Flexibility: Accommodate substrate movement

- Moisture barrier: Sealed edges, adequate DFT

- Total DFT: 3.0-4.0 mils minimum

## 5.3 Sheen Recommendations

| Sheen | Use | Durability | Touch-Up | Notes |
| --- | --- | --- | --- | --- |
| Satin | Standard interior | Good | Moderate | Most forgiving |
| Semi-gloss | Traditional, kitchens | Very good | Moderate | Shows defects |
| Gloss | High-end, entry | Excellent | Difficult | Requires Level 4 prep |

## 5.4 Blocking Prevention

Traditional latex paints are often too soft for doors, causing blocking (door sticks to weatherstripping or frame). Use urethane-alkyd hybrids, water-based urethanes, or higher resin-to-pigment ratio products.

# 6. Quality Tier Matrix

| Attribute | QT2 | QT3 | QT4 | QT5 | QT6 |
| --- | --- | --- | --- | --- | --- |
| Primer coats | 1 (may be factory) | 1 | 1-2 | 2 | 2 |
| Finish coats | 1 | 2 | 2 | 2-3 | 3+ |
| Sand between coats | No | Light scuff | Yes 220 | Yes 320 | Yes 400+ |
| Fill grain (wood) | No | No | Optional | Yes | Yes, multiple |
| Orange peel | Visible OK | Slight OK | Minimal | None | None |
| Brush marks | Visible OK | Slight OK | Minimal | None | None |
| Leveling standard | Basic | Good | Very good | Excellent | Mirror |
| Application method | Brush/roll | Brush/roll or spray | Spray preferred | Spray preferred | Spray, controlled |
| Inspection distance | 6 ft | 39" (P1) | 24" | 12" | Raking light |

Note: QT5 and QT6 finishes are achievable with skilled brush work on doors, though spray is preferred for efficiency and consistency. Method selection remains contractor discretion.

## 6.1 Quality Tier Labor Factors

| Quality Tier | Relative Labor Factor vs QT3 |
| --- | --- |
| QT2 | 0.7x (faster) |
| QT3 | 1.0x (baseline) |
| QT4 | 1.4x |
| QT5 | 1.8x |
| QT6 | 2.5x+ |

# 7. Door Handling & Setup Methods

PaintFactor Domain Knowledge: Setup method is contractor discretion based on project conditions - batch efficiency, ergonomics, available space, logistics. NOT a quality tier selection. METHOD DOES NOT CORRELATE TO QUALITY TIER.

## 7.1 Method Overview

| Method | Setup Intensity | Primary Considerations |
| --- | --- | --- |
| In-Frame (On-Hinge) | Baseline | Any quantity, any color scenario, most common |
| Chevron Stand (Vertical) | Medium | Batch processing, doors removed |
| Horizontal Handle Rack | High | Ergonomic prep, requires spray + drying areas |

## 7.2 In-Frame Method (On-Hinge) - BASELINE

When Used:

- Any number of doors - frequently used for entire houses

- Same OR different color/finish as frames

- Spray, brush, or roll application (spray requires experienced painter)

### Color/Finish Scenarios

| Scenario | Application | Masking Requirement |
| --- | --- | --- |
| Same color as frame | Brush/roll | Minimal |
| Same color as frame | Spray | Standard overspray protection |
| Different color than frame | Brush/roll | Minimal (cut in) |
| Different color than frame | Spray | Mask door OR frame (whichever painted first) |

### Hinge Handling Options

| Option | Method | Time per Hinge |
| --- | --- | --- |
| A | Mask hinges | ~5 min |
| B | Swap to dummy hinges (pre-painted sacrificial set) | ~5 min |

Dummy Hinge System: Maintain set of matching dummy hinges per common size/style. Dummy hinges accumulate paint over time (acceptable). Clean hinges stored safely during painting. Swap back after final coat cures.

### Time Factors

| Task | Time | Notes |
| --- | --- | --- |
| Floor protection | Shared | Across doors in room |
| Hinge handling (mask or swap) | ~15 min/door | ~5 min x 3 hinges typical |
| Hinge swap back to clean | ~15 min/door | Option B only |

Note: Larger doors or doors with more than 3 hinges increase hinge time proportionally.

## 7.3 Chevron Stand Method (Vertical)

When Used: Contractor preference for batch processing. Doors removed and staged vertically.

Setup Sequence:

- Remove door from frame (note hinge side, swing direction) (Note Door Location on top of door or on hinge seat)

- Remove hardware if installed - bag and label

- Install buttons on door bottom (or use shims)

- Stand doors at angles, connect with L-brackets (chevron pattern from above)

- Prep, prime, paint all accessible surfaces

- Allow dry time

- Disconnect brackets, return to field for install

## 7.4 Horizontal Handle Rack Method

When Used: Contractor preference for ergonomic prep (waist-level work reduces labor intensity). Requires adequate space for spray area AND separate drying/staging area.

(Note Door Location on top of door or on hinge seat)

Advantages:

- Prep at waist level - less labor intensive

- Doors dry perfectly flat (no sag, no drip migration)

- Stackable during dry time (handles act as spacers)

Disadvantages:

- Highest setup time

- Requires significant space (work area + separate drying area)

- Handle install/removal adds labor

# 8. Application Methods

## 8.1 Brush and Roller Technique

In occupied residential settings, brushing and rolling remain practical methods.

Panel Door Sequence (PCA recommended):

- Panels first: Cut in edges with brush, fill flat areas with small roller

- Horizontal rails: Tie in brush marks to panels

- Vertical stiles: Continuous vertical pattern

- Edges: Edge visible when door open matches adjacent face

Coat Requirements: Two coats high-quality semi-gloss or satin enamel for full hiding and durability.

## 8.2 Spray Application

Preferred for new construction or high-end renovations requiring factory-grade finish.

| Spray Type | Use Case | Overspray | Control |
| --- | --- | --- | --- |
| HVLP | Interior doors, fine finish | Lower | Higher |
| Airless | High-volume production | Higher | Lower |

Primary Advantage: Absence of brush/roller marks; perfectly smooth, uniform film.

Primary Challenge: Masking time often offsets application speed. One day of spraying may involve 6 hours masking and 1 hour painting. (This statement is true on a job that doesn’t have other paintable items that require protection.  Full interior paints benefit is spray is always option.  If repaint occupied home where brush and roll is primary application and minimal protection in place, spray doors is no feasible due to excessive masking requirements unless specifically requested by customers.

## 8.3 Cycle Time Comparison

| Factor | Hand-Painting (In Situ) | Spray (Off-Site/Area) |
| --- | --- | --- |
| Masking/Setup | 10-20 mins per door | 60-90 mins per area |
| Application (2 coats) | 40-60 mins per door | 15-20 mins per door |
| Drying/Flip Time | 4-6 hours | 1-2 hours (with fans) |
| Cleanup | 10 mins | 30-45 mins |
| Best For | Occupied homes; 1-3 doors (Can be any number of doors if spraying is not option or preferred. | New construction; 10+ doors |

# 9. Productivity Benchmarks

## 9.1 Industry Benchmarks (Resene/PCA)

| Task Description | Man-Hours | Source |
| --- | --- | --- |
| Flush door (interior, new, 2 coats) | 3.60 | Resene Table |
| Flush door (interior, repaint, 2 coats) | 2.90 | Resene Table |
| Flush door (interior, sprayed) | 3.00 | Resene Table |
| 6-Panel door (brush/roll, 1 side) | 0.50-0.75 | Field Average |
| Exterior entry door (both sides) | 1.00-3.00 | Industry Average |
| Fire door (high complexity) | 4.90 | Resene Table |
| Bi-fold doors (paired) | 3.60 | Resene Table |

Note: Resene man-hours represent total cycle time including prep, prime, and all finish coats per door.

## 9.2 Complexity Modifiers

| Factor | Modifier | Description |
| --- | --- | --- |
| Louvered door | +10-15% | Slat surface area, recessed edges |
| Overheight (>7 ft) | +18-25% | Ladder use, physical fatigue |
| French door (glazed) | +80-120% | Glass masking intensive |
| Panel door vs flush | +15-35% | Detail work per panel count |

# 10. Hardware & Hinge Handling

## 10.1 Hinge Time Reference

| Hinge Count | Mask or Swap Time |
| --- | --- |
| 2 hinges | ~10 min |
| 3 hinges (standard) | ~15 min |
| 4 hinges | ~20 min |
| 5+ hinges (heavy/tall doors) | ~5 min per hinge |

## 10.2 Hardware Handling (Removal Methods)

| Task | Time | Notes |
| --- | --- | --- |
| Hardware removal | 5-10 min | Lockset, strike plate |
| Bag and label | ~2 min | Per door |
| Hardware reinstall | 10-15 min | Alignment, adjustment |

## 10.3 Edge Painting Rule

Professional practice: Edge visible when door is open should match adjacent face.

| Door Position | Hinge-Side Edge | Lock-Side Edge |
| --- | --- | --- |
| Opens into room | Match hallway side | Match room side |
| Opens out of room | Match room side | Match hallway side |

# 11. Common Defects & Mitigation

| Defect | Cause | Prevention | Remediation |
| --- | --- | --- | --- |
| Runs/sags | Over-application | Proper mil thickness, light coats | Sand flat, recoat |
| Orange peel | Wrong tip/pressure | Adjust equipment, distance | Sand 320+, recoat |
| Brush marks | Wrong brush, technique | Quality brush, lay-off strokes | Sand, respray |
| Edge failures | Poor edge seal (MDF) | Seal edges before prime | Strip, seal, reprime |
| Grain telegraph | Insufficient fill | Multiple fill/sand cycles | Sand back, fill, recoat |
| Adhesion failure | Inadequate prep | Proper prep, adhesion primer | Strip, start over |
| Blocking | Soft coating | Use urethane-alkyd, full cure | Recoat harder enamel |
| Surfactant leach | Low temp/high humidity | Apply in proper conditions | Wash off, allow cure |
| Fisheying | Contamination | Thorough cleaning, TSP wash | Strip, clean, restart |

## 11.1 Environmental Application Windows

| Condition | Minimum | Maximum | Notes |
| --- | --- | --- | --- |
| Temperature | 50°F (10°C) | 90°F (32°C) | Water-based coatings |
| Humidity | 20% RH | 85% RH | Affects cure time |
| Direct sunlight | Avoid | Avoid | Rapid dry, poor leveling |

# 12. Data Gaps & Cross-References

## 12.1 Identified Data Gaps

- Specific door removal/reinstall time per setup method (chevron, horizontal)
Hardware removal 20min per door. 
Prop up with buttons or shims, stand doors and install brackets 2-5min a door)
Remove Brackets and relocate door 2-5min a door
Reinstall hardware, install door 10-20min a door

Horizontal-
Same hardware situation uninstall reinstall
5-10 min a door to install handles / brackets, same with uninstall similar time

- Detailed bifold track painting considerations
Mask track when spraying in place or spraying frame. 
Remove or mask track rollers on door when spraying / staging.

- Louver door spray technique specifics
Better to spray 3 light finish coats on louver doors than 2 normal coats and risk drips on louvered portion and sand and remove takes extremely long time.

- Multi-color door scenarios (different color per side) productivity impact
edge work masking on leaf edge.  Standard mask and cut rates

- Sidelite and transom integration with entry door specs
Count sidelite and transom as separate openings, window opening rate applies with same scrutiny from window doctrine

- Garage entry door (fire-rated) specific requirements
Same exterior rules, tape door lable on hinge side to avoid covering with paint but same scrutiny as others exterior grade

## 12.2 Cross-Reference Requirements

- Validate QT tier definitions align with existing PaintFactor tiers

- Confirm production rates align with PaintFactor_Production_Rate_Reference.md

- Verify complexity modifiers match existing height/profile modifiers

- Map door types to PaintScope quantity keys (PS_SURFACE_EA.DOOR_*, etc.)

## 12.3 Sanding Grit Progression Note

Review Action: The sanding grit progression (220 at QT4, 320 at QT5, 400+ at QT6) should be considered for adoption into Fine Finish Doctrine and Quality Tier Doctrine as a standardized reference.

# Document Approval

| Field | Value |
| --- | --- |
| Prepared By | NotebookLM Research Agent + Claude |
| Reviewed By |  |
| Review Date |  |
| Approved By |  |
| Approval Date |  |
| Promoted to Doctrine | [ ] Yes [ ] No - Requires Revision |

--- End of Document ---