Mask Level Decision Matrix

Fill in each cell with one of: none / edge / partial / full / encapsulate. Add notes per row where the answer depends on something not captured by the columns. Add rows wherever a real-world scenario doesn't fit.

Mask Level Legend

| Mask Level | Definition |
| --- | --- |
| none | No protection — surface is in scope or doesn't need it |
| edge | Tape line only — perimeter strip / trim line tape |
| partial | Drop cloth or plastic extending out from edge (~3 ft strip for floors; perimeter for ceilings) |
| full | Loose drape covering full surface (untaped). N/A for ceilings (gravity). |
| encapsulate | Taped + sealed tight — full coverage with edges sealed |
| Edge+ | Edge tape line with a second pass of other Mask Level. Edge+ Partial, Edge+ Full, Edge+ Encapsulate. (Usually for when spraying walls and ceilings. The Edge tape line is for accuracy of the edge coverage and protection. Usually using a tape that has a good edge seal like green frog tape. Delicate surfaces like cabinets receive a delicate surface tape along edge. All other masking is applied over top of the delicate surface tape. ) |

Table 1 — Floor Mask Level

Rows = painting scope × application method. Columns = floor type. Each cell = mask level when those conditions match. Subfloor + Concrete columns included for completeness (likely 'none' or 'edge' in most cases — confirm).

| # | Painting Scope | Method | Subfloor | Concrete | Hardwood | Tile | Carpet | LVP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Ceiling only | Brush/Roll | None | Full – 3Mil Plastic or Drop Clothes | Full Drops | Full Drops | Full Drops | Full Drops |
| 2 | Ceiling only | Spray | None | Full – 3Mil Plastic or Drop Clothes | Full Drops | Full Drops | Full Drops | Full Drops |
| 3 | Walls only | Brush/Roll | None | Perimeter Drops | Perimeter Drops | Perimeter Drops | Perimeter Drops | Perimeter Drops |
| 4 | Walls only | Spray | None | Full Drops or Encapsulate 3Mil Plastic or Rosin Paper / Construction Paper | Perimeter Drops or Rosin / Construction Paper | Perimeter Drops or Rosin / Construction Paper | Perimeter Drops or Rosin / Construction Paper | Perimeter Drops or Rosin / Construction Paper |
| 5 | Trim only | Brush | None | Optional Edge+ Default - Perimeter Drops or Rosin | Optional Edge+ Default - Perimeter Drops or Rosin | Optional Edge+ Default - Perimeter Drops or Rosin | Optional Edge+ Default - Perimeter Drops or Rosin | Optional Edge+ Default - Perimeter Drops or Rosin |
| 6 | Trim only | Spray | None | Edge+ Encapsulate – 3Mill Plastic or Drop Clothes | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate |
|  | Baseboard Only | Brush | None | Optional – Default=Perimeter | Optional – Default=Perimeter | Optional – Default=Perimeter | Optional – Default=Perimeter | Optional – Default=Perimeter |
|  | Baseboard Only | Spray | None | Edge+ Perimeter | Edge+ Perimeter | Edge+ Perimeter | Edge+ Perimeter | Edge+ Perimeter |
|  | Door Casing Only or Door Frame only | Brush | None | Drops at location | Drops at location | Drops at location | Drops at location | Drops at location |
|  | Door Casing Only or Door Frame only | Spray | None | Edge+ (@ base of frame/casing) Masing. UOM /EA. 5min install 1min teardown | Edge+ (@ base of frame/casing) Masing. UOM /EA. 5min install 1min teardown | Edge+ (@ base of frame/casing) Masing. UOM /EA. 5min install 1min teardown | Edge+ (@ base of frame/casing) Masing. UOM /EA. 5min install 1min teardown | Edge+ (@ base of frame/casing) Masing. UOM /EA. 5min install 1min teardown |
| 7 | Ceiling + Walls | All brush/roll | None | Full – 3Mil Plastic or Drop Clothes | Full Drops | Full Drops | Full Drops | Full Drops |
| 8 | Ceiling + Walls | All spray | None | Encapsulate | Encapsulate | Encapsulate | Encapsulate | Encapsulate |
| 9 | Ceiling + Walls | Ceiling spray, walls brush | None | Full – 3Mil Plastic or Drop Clothes | Full Drops | Full Drops | Full Drops | Full Drops |
| 10 | Ceiling + Walls | Walls spray, ceiling brush (rare) | None | Full – 3Mil Plastic or Drop Clothes | Full Drops | Full Drops | Full Drops | Full Drops |
| 11 | Walls + Trim | All brush | None | Optional Edge+ Default Perimeter | Optional Edge+ Default Perimeter | Optional Edge+ Default Perimeter | Optional Edge+ Default Perimeter | Optional Edge+ Default Perimeter |
| 12 | Walls + Trim | All spray | None | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate |
| 13 | Walls + Trim | Trim brush, walls spray | None | Optional Edge+ Default = Perimeter | Optional Edge+ Default = Perimeter | Optional Edge+ Default = Perimeter | Optional Edge+ Default = Perimeter | Optional Edge+ Default = Perimeter |
| 14 | Walls + Trim | Trim spray, walls brush | None | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate |
| 15 | Ceiling + Trim | All brush | None | Optional Edge+ Default = Full | Optional Edge+ Default = Full | Optional Edge+ Default = Full | Optional Edge+ Default = Full | Optional Edge+ Default = Full |
| 16 | Ceiling + Trim | All spray | None | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate |
| 17 | Ceiling + Walls + Trim | All brush | None | Optional Edge+ Default = Encapsulate | Optional Edge+ Default = Encapsulate | Optional Edge+ Default = Encapsulate | Optional Edge+ Default = Encapsulate | Optional Edge+ Default = Encapsulate |
| 18 | Ceiling + Walls + Trim | All spray | None | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate |
| 19 | Ceiling + Walls + Trim | Trim brush, walls + ceiling spray | None | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate |
| 20 | Ceiling + Walls + Trim | Trim spray, walls + ceiling brush | None | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate |
| 21 | Full (CWT + doors + windows) | All brush | None | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate |
| 22 | Full | All spray | None | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate | Edge+ Encapsulate |
| 23 | Custom — see notes |  | None |  |  |  |  |  |

Table 2 — Wall Mask Level

Only applies when walls are NOT in paint scope. When walls ARE in scope, mask level is always 'none' (you paint them, no overspray-on-self concern). Wall material is assumed to be finished drywall — flag in notes if a row should differ for bare drywall, masonry, or wood paneling.

| # | What IS being painted (walls NOT in scope) | Method | Wall mask | Notes |
| --- | --- | --- | --- | --- |
| 1 | Ceiling only | Brush/Roll | Full | Drape Walls |
| 2 | Ceiling only | Spray | Edge+ Full | Tapeline Edge at Wall-ceiling line |
| 3 | Trim only | Brush | Optional Edge Default = None | Cut in trim to wall |
| 4 | Trim only (no crown) | Spray | Edge+ Perimeter |  |
| 5 | Trim only (incl crown) | Spray | Edge+ Perimeter |  |
| 6 | Ceiling + Trim | All brush | Full Wall | Paint Ceiling First, Tear down and then paint trim with trim wall mask recomendation |
| 7 | Ceiling + Trim | All spray | Edge+ Full | Edge at both Wall-Ceiling Line Wall-Trim Line, Or sequencially do ceiling then trim |
| 8 | Ceiling + Trim | Ceiling spray, trim brush | Edge+Full |  |
| 9 | Doors + windows only | Brush | Optional Edge Default = None |  |
| 10 | Doors + windows only | Spray | Edge+ Perimeter |  |
| 11 | Custom — see notes |  |  |  |

Table 3 — Ceiling Mask Level

Only applies when ceiling is NOT in paint scope. When ceiling IS in scope, mask level is always 'none'. Ceiling has no 'full' option — gravity prevents draping. Available levels: none / edge / partial / encapsulate.

| # | What IS being painted (ceiling NOT in scope) | Method | Ceiling mask | Notes |
| --- | --- | --- | --- | --- |
| 1 | Walls only | Brush/Roll | None | Cut In wall @ceiling line |
| 2 | Walls only | Spray | Edge+ Perimeter |  |
| 3 | Trim only — no crown | Brush | None |  |
| 4 | Trim only — no crown | Spray | Spot | Above Windows and Doors Per /EA opening 1min install 1min tear down |
| 5 | Trim only — incl crown | Brush | Optional Edge Default = None |  |
| 6 | Trim only — incl crown | Spray | Edge+ Perimeter |  |
| 7 | Walls + Trim | All brush | None | Cut In wall @ceiling line |
| 8 | Walls + Trim | All spray | Edge+ Perimeter |  |
| 9 | Walls + Trim | Trim brush, walls spray | Edge+ Perimeter |  |
| 10 | Walls + Trim | Trim spray, walls brush | Spot | Cut in wall @ceiling line, Spot above windows and doors per /EA opening 1min install 1min tear down |
| 11 | Doors + windows only | Brush | None |  |
| 12 | Doors + windows only | Spray | Spot | Cut in wall @ceiling line, Spot above windows and doors per /EA opening 1min install 1min tear down |
| 13 | Custom — see notes |  |  |  |

Notes / Edge Cases

Capture anything that doesn't fit cleanly in the matrix above:

•

•

•

Open questions to flag

•  Does floor type override painting scope (e.g., spraying anything on hardwood always = encapsulate, regardless of scope)?

•  When trim is sprayed, does the spray pattern reach all 3 surfaces (floor + wall + ceiling), or only floor + wall?

•  Are there 'minimum' protections that always apply regardless of scope (e.g., always edge-tape floor at trim line)?

•  Does "Custom" painting scope use the strictest level across all active substrates, or pick a default?